The Ovonic Cognitive Computer

A proposal for a creating a cognitive chip

Energy Conversion Devices, Inc.

2956 Waterview Drive

Rochester Hills, MI   48309-3484

EXECUTIVE SUMMARY

Many complex tasks readily and intuitively performed by humans remain difficult or impossible for current computers, and conceptual, physical, and economic barriers prevent their reaching such a goal.  A paradigm change is needed, embracing new concepts and materials, to achieve the required massive intrinsic parallelism.  Small, thin-film, scalable, fast, chalcogenide-based Ovonic devices with plasticity, nonvolatility, multistate capability, and biomimetic neurosynaptic behaviors have been successfully demonstrated by Energy Conversion Devices, Inc. (ECD) (1-4).  The Ovonic cognitive devices are based on the amorphous chalcogenide technology pioneered at Energy Conversion Devices, Inc. (ECD) by S. R. Ovshinsky, inventor of phase-change devices, including phase-change optical memory and the Ovonic Universal Memory, as well as the Ovonic threshold switch in its 2-and 3-terminal configurations.  ECD proposes herein to demonstrate cognitive behaviors of neural networks and more general cognitive computer configurations (5) and, despite the impracticality of quantum computing, to emulate various quantum computer functionalities by fabricating configurations of individual Ovonic devices in a hybrid chalcogenide-on-silicon x-y array for and on which algorithms, architecture and software can be implemented.  Our goal is the creation of a family of unprecedentedly powerful cognitive computers. The initial product would consist of a multiple 128x128-array chip set on a card readily installed in a personal computer together with the relevant software transparently implementing our powerful new proprietary algorithms, addressed to diverse markets.  It sets the stage for subsequent chip sets of 104 x 104 arrays with 108 neurosynapses achievable with current technology.  Complex problems normally reserved for supercomputers would then become accessible to personal computers.  We have demonstrated similar cognitive functionality when using optical energy to address the devices, enabling future hybrid electro-optical circuits.

I.  INTRODUCTION

Our goal is the creation of a family of unprecedentedly powerful cognitive computers.

As a first step towards that goal, we propose to create a family of Ovonic cognitive neural networks consisting of 128 x 128 = 16,384 nodes.  At each node there is an Ovonic cognitive device functioning as a neuronal synapse.  Hybrid chalcogenide-on-silicon x-y arrays will be fabricated on which Ovonic device materials and configurations can be optimized and for which algorithms, architectures, and software can be implemented.  The technology to do so is currently available.  The initial product would be a family of multiple 128x128-array chip set on a card readily installed in a personal computer together with the relevant software transparently implementing our powerful new algorithms, the members of which are addressed to diverse markets. The stage would then be set for a more powerful family of computers based on chip sets of 104 x 104 arrays of 108 nodes. Fast, massively parallel networks of that size would have an enormous and immediate range of cognitive applications of which data mining; pattern recognition; medical diagnosis; intelligent information organization, searching, and analysis are but a few (6).

Such networks could be configured to carry out in a single step a range of actual mathematical operations which in conventional computers require many repeated operations, greatly accelerating the solution of complex problems.

A network could be configured as a cognitive search engine.  At the 104 x 104 scale, for example, it would process 80 bit words 100 times faster than a Pentium IV can process 64 bit words.  The search engine would constitute a fast associative memory.

A network could be configured as any one of a family of neural networks.  There would then be 128 additional Ovonic cognitive devices functioning as output neurons of the 128 x 128 network.

The network could as well be configured to give full biomimetic neuronal functionality in local circuits connected into networks of neurons, a totally new achievement.

Used in combination as a network of networks in a chip set, such fast, massively parallel networks would have an enormous and immediate range of applications.  Adapting software to use specifically with each type of network in the set and with the set as a whole would give orders of magnitude improvement of performance.

For example, we have shown by simulations that when a conventional computer model of a neural network is used for the solution of complicated boundary value problems, spectral codes adapted for our applications are 10 to 100 times faster than the ubiquitous finite element codes.  Implementing the conventional neural network algorithms in our proposed hardware would give a comparable additional increase in speed.  Adapting the algorithms to our unique devices, as described herein, would give a further significant increase in speed.  Finally, using our mathematical processing networks to carry out the intermediate processing steps would add another burst of speed.  We estimate that overall acceleration by 106 or more is an achievable goal.

Relatively small neural networks implemented in silicon-based hardware but primarily in software have been in commercial use for a decade and a half to solve small problems in a wide range of contexts.  However, “artificial neural networks also scale notoriously badly, so most successful simulations have usually used networks with fewer than 1,000 ‘neurons’ in contrast to the 100,000 neurons contained in each cubic millimeter of neocortex”(7).  Similarly, neural networks built in silicon by CMOS technology scale badly as well, with a large footprint for each node and long cycle times, e.g., milliseconds for 10,240 nodes (8).  It is this scaling problem which has been a major impediment to realizing commercially the potential power of neural networks.

We defeat this scaling problem through the use of Ovonic cognitive devices (1-5, 9), which, as with all Ovonic devices, have the required scaling properties (10-11).  These would operate as individual devices, in their multistate mode as synapses at each node and in their cognitive mode as output neurons to provide the nonlinear transfer function required by neural networks.  The Ovonic cognitive devices, based on the amorphous chalcogenide technology pioneered at Energy Conversion Devices, Inc. (ECD) by S. R. Ovshinsky, who invented phase-change devices, including optical memory and the Ovonic Universal Memory, as well as the Ovonic threshold switch in its 2-and 3-terminal configurations.

The Ovonic cognitive devices are small, eliminating the footprint problem, and fast, eliminating the speed problem (10-6 sec is achievable in a 104x104 array).  Their plasticity, nonvolatility, multistate capability, and switching behavior mimic neurosynaptic behavior, greatly increasing the power and utility of the networks and opening up further possibilities of continued development.  Ovonic cognitive devices share with biological neurons a character noted by Katz:  “Each nerve cell, in a way, is a nervous system in miniature.”(12).

Moreover the Ovonic threshold switch, both in its 2-terminal and 3-terminal configurations, is the fastest known room-temperature device, sustaining 50 times more current density than the best transistors in its conducting state, yet it maintains the submicron footprint and the nanoscale potentiality of the cognitive devices.  It can thus eliminate the large transistors from peripheral drive circuitry and reduce the footprint of the network further.

Our program has four interwoven components which proceed in parallel over the initial three-year period.  First, the Ovonic cognitive devices are to be optimized both in materials and configurations.  Second, software development of algorithms, architecture, and code will be carried out at ECD.  The neuromimetic properties of the Ovonic cognitive devices provide unique opportunities to merge the software and hardware.  Third, specifications for a silicon-substrate chip will be defined at ECD and sent outside for design and fabrication.  It will contain all subsidiary circuitry needed for the x-y array of Ovonic devices.  It will be flexible, allowing reconfigurability of the hardware.  Fourth, the Ovonic neural network will be fabricated at ECD on the silicon chip using our unique amorphous chalcogenide fabrication facilities.

The demonstration chip set, with the 16,384 nodes of its network operating in massive parallelism and with the cognitive capabilities of its Ovonic devices would have many commercial applications.  We therefore anticipate prompt commercialization of the 128 x 128 prototype.  We propose that the next step in scaling up the network is a 104 x 104 array.  Moreover, because the devices and the networks will be commercially fabricated by the same technology as Intel, STM, BAE, Elpida, Samsung, Hitachi, Toshiba and others use for the Ovonic Universal Memory, we anticipate future arrays of 1010 synapses.  Going beyond 1010 synapses will be enabled by multilayer technology, for which ECD’s Ovonic chalcogenide technology is well suited.

II.  THE OVONIC DEVICES

A.  THE OVONIC COGNITIVE DEVICE:  The Ovonic cognitive device is based on Ovshinsky’s atomically-engineered multicomponent chalcogenide phase-change materials.  Its physical configuration is a narrow channel of the phase-change material within a thin insulating film.  ECD has in-house fabrication technology capable of reaching the nanoscale, so that density limitations are imposed only by the foundry which fabricates the Si-substrate chip.

A single Ovonic cognitive device has two cognitive modes of operation, as shown in Figure 1.  The left panel illustrates operation in the cognitive register mode.  The phase-change material is initially amorphous in the reset state with a high channel resistance.  An applied electrical pulse of suitable amplitude and duration induces partial crystallization of the phase-change material with little effect on the resistance.  Repeated application of the pulse increases the degree of crystallization until a continuous crystallization path is formed and a dramatic drop of resistance results, much as a real neuron “fires” when its threshold is reached.  This sigmoidal response of resistance to pulse number makes the Ovonic cognitive device in its cognitive mode ideal for the output neuron of a y-line of the neural network.  In this mode, the device can be designed to show clear change in resistance after each pulse, expanding its capability for multi-state storage, or it can be designed to show very little change except after the “firing” pulse, so that the device can also be used for secure encryption, since the intermediate-state information is not available by any forensic means.

The right panel of Figure 1 illustrates the multistate cognitive mode of operation.  Initially in the set state, partial amorphization and a resistance increase is effected by a reset pulse of greater amplitude than the set pulses of the cognitive mode.  Reset pulses of progressively greater amplitude increase amorphicity and the resistance until the amorphous reset state is regained.  This programmable resistance makes the Ovonic cognitive device ideal for the weighting element of a neural network, functioning as a synapse between an x- and a y-line of the network.  One type of device thus provides any neurosynaptic functionality required by any device in the Ovonic cognitive neural network.

Figure 1 - Resistance characteristics of a Single Ovonic Cognitive Device.  The cognitive amorphous pre-threshold synaptic regime (left side) culminates in a percolative phase change to crystalline material, functionally equivalent to neurosynaptic switching.  The resistance change accompanying the transition to the crystalline regime can provide readout and transferring of a completed signal to other devices.  The leftmost and rightmost data points of (the high resistance endpoints) both correspond to material that is substantially amorphous, and the material becomes increasingly crystalline toward the center of the figure, with the lowest resistance states having the greatest crystallinity.  The right side is the multi-state cognitive regime.  One should look upon the left side as being either standalone, summing up the synaptic information, or united with the activities of the right side.

In the cognitive mode, the Ovonic cognitive device can carry out all arithmetic operations.  Modular arithmetic can be done in a controllable base n, the number of pulses required to reach the set state, which in turn leads to an efficient factoring algorithm with intrinsically parallel properties and to multistate logic.  The hybrid chip will be designed with sufficient flexibility in its architecture to allow demonstration of an Ovonic cognitive computer designed to exploit these remarkable properties, as well as fabrication of an Ovonic cognitive neural network.  The Ovonic cognitive computer is able to operate in the binary mode, higher modes (n > 2) and mixed modes with different n’s for the registers and for the multistate devices in the network.

The goal for the number of pulses needed for currently existing devices to set reliably in the cognitive mode will be determined by the results of our simulations.  Similarly, the number of resistance states in the multistate mode to be programmed and read reliably will also be determined by our simulations.  Specifications for reliability and stability will be achieved by optimization of device materials and configurations.  We envisage programming currents initially in the range of 0.5-1mA, requiring device diameters of 200nm or less with stable resistive contacts.  Scalability  of Ovonic devices with its concomitant increase of density and speed and decrease of programming current is well accepted (11).

B.  THE OVONIC THRESHOLD SWITCH:  In contrast to the Ovonic cognitive device, which is based on the Ovonic phase-change materials, the Ovonic threshold switch is based on multicomponent chalcogenide semiconductors atomically engineered to remain stable in the amorphous phase, following Ovshinsky’s design principles.  The Ovonic threshold switch retains high resistance until a threshold voltage is reached, when it switches at sub-picosecond speeds to a low resistance state, reversibly and symmetrically, independent of voltage sign.  It remains in that conducting state until the current falls below a holding value.  The current density presently achieved is 30 times higher than that of the best transistors.  We shall therefore use Ovonic threshold switches in place of the large transistors required to generate the above programming currents, thereby substantially reducing the footprint of the control circuitry of the network, as shown in Figure 2.  The 3-electrode Ovonic threshold switch has been demonstrated, showing modulation and control of the threshold voltage and, remarkably, elimination of the holding current.

III.  THE HYBRID COGNITIVE NETWORK CHIP

We propose to have an ASIC wafer specifically designed to our specifications and made and processed by an outside foundry.  The resulting CMOS chip will have a simple and flexible structure so that the widest possible variety of neural network and cognitive computer networks can be tested.  The network structure will be completed in our facility with the addition to the chip of the Ovonic chalcogenide technology.  It will have 128 rows and 128 columns as indicated schematically in Figure 3.  This array size was chosen to make networks of size suitable for effective and timely demonstration of the technology’s potential.

In the particular case of the Ovonic cognitive neural network, at each intersection the rows and columns are connected by 128 x 128 isolated Ovonic cognitive devices operating in the multistate mode (synapses), as indicated in Figure 3 by an encircled X.  Row circuitry will allow input from an external input vector or feedback from the columns into the rows.  Column circuitry will allow sensing of the read signal along each column and will have separate Ovonic cognitive devices (neurons) to implement neuronal functionality.  In the case of the Ovonic cognitive neural network, these would operate in the register mode and provide the requisite sigmoidal transfer function required by neural networks.  The rows will have pulse-generating circuitry including Ovonic threshold switches for programming the synapses as will the columns for programming the neurons, cf Figure 2.  A flexible set of control signals will set pulse parameters, route signals, and provide for varied implementation of Ovonic cognitive neural networks and of cognitive function. The non-volatile nature of the structural changes in the devices means that not only is the state of all the devices retained in the event of power loss, but also that a calculation started at one point in time can be completed at any time later.

The specifications from which outside silicon architects will design the CMOS chip will be established by our simulation of a range of candidate networks, of both Ovonic cognitive neural network and Ovonic cognitive computer types.  Completed chips will be tested and characterized and array specifications set for fabrication of demonstration chips.

Figure 3 - Proposed structure of test chip

IV.  SOFTWARE CONSIDERATIONS

The design of the Ovonic cognitive neural network and Ovonic cognitive computer networks, the creation of the algorithms for network functioning, and the optimization goals for the Ovonic cognitive device and Ovonic threshold switch are intimately interconnected.  Simulation on conventional computers allows us to break though this web of interrelations.  For example, the accuracy of the Ovonic cognitive neural network will depend on the number of discrete resistivity states in the multistate mode, the number of discrete resistivity states in the registers, the minor fluctuations in the corresponding resistivity values during operation, and the size of the networks.  Through sensitivity analyses for the first three and scaling analysis for the last, we shall establish minimum acceptable values for each in relation to standardized tasks.  Thus goals will be established for device operation, and applications will be established for which the 128 x 128 array is well suited.  A likely result of the simulations will be a clear demonstration that before using existing algorithms for cognitive neural networks, they will have to be adapted to the Ovonic cognitive device characteristics.  Most are not very suitable for implementation in hardware because of the above issues (13).  Thus, even though neural networks have a long history, specific algorithms will be necessary for our Ovonic cognitive neural network.

For the Ovonic cognitive computer, one base to build is the set of novel algorithms we have already established for arithmetic operations, factoring, multistate logic, more complex mathematical one-step operations, distance measurement, and searching.  A second base to build on is the adaptation of our existing powerful proprietary conventional software for complex boundary-value problems, clustering, and pattern recognition to run on our cognitive networks.  This activity will continue in order to establish the design specifications of the flexible CMOS chip and explore the opportunities created by the unique capabilities of the Ovonic cognitive devices and the Ovonic threshold switch.

Based on the experience with simulations, we shall establish a mathematical framework for describing our networks which will identify algorithms, software architecture, and code writing in coordination with specification of the CMOS chip architecture and that of the superposed Ovonic network.  One candidate formalism is functional chip design through use of functional equations.

One promising avenue for substantially decreasing sensitivity to the discreteness of the resistance values of the Ovonic cognitive devices and to their fluctuations proceeds by representing data by pseudo random points in a high dimensional space (14).  Consider two distinct input data represented by two typical points in that space.  When operated on by our network, the statistical fluctuations smear the representative points of the output data.  Nevertheless, we have shown that the likelihood that the output data will be mistakenly identified as belonging to the same input data decreases dramatically with increase in the dimension of the space in a manner insensitive to the discreteness of the resistance.

V.  WORK PLAN

A.  Algorithms, Architecture And Software:  Learning algorithms appropriate for Ovonic cognitive devices will be developed prior to design of the 128 x 128 array.  Existing learning algorithms need to be adapted so as to increase the tolerance to component inaccuracies while retaining a satisfactory overall network performance.  Improved new algorithms will be developed.  Algorithms will later be fine-tuned on the actual chips to promote fault tolerance using promising methods proposed in the literature and our own novel schemes.

B.  Device Optimization:  The basic technology for building individual Ovonic cognitive devices has been developed by ECD.  In this program we will optimize the performance of the devices both in the register and the programmable resistance (multistate) modes.  We will develop a simplified chip design that does not include the silicon circuitry and has Ovonic devices spaced to accommodate wafer probes.  We will address the reliability and stability of the resistance states, and make arrays which will facilitate statistical evaluations of device characteristics.

C.  Chip Development:  The 128 x 128 network CMOS chip will have a flexible structure so that the widest possible variety of neural and cognitive network structures can be pursued.  It will have 128 rows and 128 columns of isolated chalcogenide Ovonic memory elements.  A flexible set of control signals is provided to enable a variety of functional implementations.  The chip architecture will follow from the learning algorithms.  Most of the silicon work will be done under subcontract to an organization specializing in such architectural work.

Fabrication on the CMOS chip of the Ovonic cognitive neural network and the network sub-structures via our chalcogenide technology will be performed at ECD with thin-film deposition and micro-lithography patterning techniques both familiar and similar to most conventional semiconductor manufacture.  Specifically, the custom silicon driver wafers will have been designed and laid out with an open architecture that facilitates maximum wiring flexibility for multiple neuronic configurations at ECD.  This design work will be done by a subcontractor, working with ECD.  The design needs to be customized and optimized for use with the switching and cognitive alloys. Using the design, wafers with CMOS circuit components will be fabricated at a silicon foundry.  ECD has a clean room facility that is used to process our chalcogenide cognitive and switching devices.  In our facility we would then complete the Ovonic chip by making chalcogenide cognitive devices and threshold switches, followed by the final metallization step.

D.  Implement Algorithms and Demonstrate Functionality:  The algorithms will be implemented on the chip using programs to generate pulses from the on-chip pulse generators.  We plan to exploit the flexibility we design into the chip to be able to adjust synaptic weights using both the mode of programmable resistance and the Ovonic cognitive operation.  Test patterns will be inputted, and then the circuit configuration will be adjusted according to the output vectors.  Successive implementation of our novel learning algorithms will continually increase the accuracy of the performance of the circuit.

The timing of the tasks is shown in the Gantt Chart.

VI.  APPLICATIONS

Applications

Possible applications of a chip set incorporating cognitive neural networks, mathematical networks, and search engines of which we propose to build 128 x 128 prototypes are numerous and wide ranging.  A few specific examples are data mining; image and sound compression; pattern recognition; medical diagnosis; factoring; and intelligent information organization, searching, and analysis.  They fall into two broad categories, information processing and the simulation and optimization of dynamic systems, although aspects of each are present in the other.  We interpret dynamic in the broadest possible sense, incorporating repeated operations such as iteration, repeated events in real time, and processes continuous in time.  Within each category there are far too many possibilities to enumerate here, and we give only a few prominent examples.

In the dynamic systems category, two quite different applications illustrate the range of possibilities.  The first is speech recognition.  Current digital programs work with high accuracy for unknown speakers with a limited preset vocabulary in a stimulus-response mode.  Digital programs which are trained to recognize the speech of a single individual work with less accuracy but with a much larger vocabulary.  Within the context of speech with pauses between each word, they can recognize up to 20,000 words.  Most such programs simulate neural networks in conventional computers.  Our cognitive neural and other networks will greatly increase hearing capability through their superior scalability to higher densities as well as superior speed via hardware architecture adapted to the task as opposed to the hardware of a general purpose computer, via our novel software adapted to the hardware as well as to the task, and via intrinsically faster devices.  The goal would be understanding a 50,000 word vocabulary of an unknown speaker.  It is a holy grail, and we believe it reachable via our proposed technology when scale up is achieved.

The second concerns the solution of complicated boundary value problems.  Finite element programs are in routine use in all fields of engineering for design and for analysis, and similarly in architecture.  Our current software for use with conventional computers is 10 to 100 times faster for the complicated thermal problems we have studied, and the speed up is generic and not related to the specific problem.  Moreover, opportunities for further speed up of our programs exist by intensive substitution of single step network operations for operations which scale as N or N2 in conventional computers, where N is a number measuring the size of the data operated on.  Substituting the hardware and software of our cognitive networks for the simulated neural networks and conventional codes we have used thus far would yield a further dramatic speed up estimated to be on the scale of 106.  Thus, a substantial market would exist for a 128 x 128 cognitive chip set once commercialized subsequent to the completion of this proposal.  For example, a major automobile maker now bases its engine designs on a finite-element model so computationally complex that it cannot be used for design optimization even when run on their supercomputer.  Instead, some performance information is generated and a computationally simple surrogate model is fitted to that paucus data.  With our goal of a factor of 106 speed up via our cognitive networks, the time scale would be reduced from months to seconds, permitting full use of the design model and avoiding the use of much less accurate surrogates.

The information processing category is vast.  Examples are static language processing, e.g. translation as opposed to speech recognition in real time; intelligent information organization, searching, and processing; pattern recognition; etc.   A fast processor specialized for searching and recognition could greatly accelerate such information processing.  We have created an efficient new algorithm and a concept for a network of Ovonic cognitive devices on which it can be implemented.  With our 104 x 104 network and 10 to 16 states per synapse, we project that our search rate would be equivalent to about 5 x 1011 80-bit words per second.  This rate is to be compared with a word rate which we believe to be of about 5 x 109 64-bit words per second for a Pentium IV processor, a significant acceleration.  However, this hundred-fold acceleration of a simple word-by-word comparison, though important, is not the most significant improvement achievable with the proposed technology.  With a hierarchical integration of modules consisting of our neural networks, mathematical networks, and search engines, we could construct a hierarchically organized taxonomic or cladistic data base much like the Linnean classification system and its successors.  Highest level features would be extracted initially from input data, and a clustering analysis performed to establish the highest level category and so on down the hierarchy.  Such a system could be used as a powerful associative memory, for pattern recognition, for classification, for such bioinformatics as genomics and proteomics and for many other applications, it would advance us towards cognition.

Another information processing application is secure encryption.  Information can be stored in our Ovonic cognitive device in the cognitive mode, free from forensic attack with no possibility of retrieval without the key.  The minor resistivity changes prior to the set threshold are completely hidden, masking the number of applied pulses and the number of pulses to threshold.  Much information is thus encrypted in the number of pulses in each device.  Pulse shape provides further degrees of freedom for encryption.

VII.  ACCOMPLISHMENTS; NEXT STEPS

At the end of the three-year program, we will have a working prototype of the first product including all necessary functions.  Therefore, this will be the first commercial product.  We summarize some of the principal accomplishments and follow-up actions in the next two subsections.

A.  ACCOMPLISHMENTS:

1.	The Ovonic cognitive devices and 2- 3-terminal Ovonic threshold switches will be optimized for incorporation in Ovonic cognitive neural nets and Ovonic cognitive computers.  The optimizations will differ according to the application with specific optimizations for specific networks.

2.	A flexible hybrid chip architecture will be developed which admits a range of network fabrications including different neural networks and cognitive computers.

3.	Algorithms will be developed and implemented in software and hardware which exploit optimally the unique properties of the Ovonic devices.

4.	Applications for both the cognitive neural network and the cognitive computer will be identified which are ready for commercialization at the scale of the 128 x 128 demonstration chip set.  That is, the demonstration chip set will become a commercial product.

5.	A program for scaling up to the 104 x 104 chip will be defined.

B.  NEXT STEPS

1.	Preparation for the production of the 128 x 128 chip set.

2.	Production of demonstration chips for initiation of marketing.

3.	Initiation of a two-year program for development of the 104 x 104 chip.

4.	Initiation of a three-year program for increasing chalcogenide content of the hybrid chip, replacing silicon-based functionality with Ovonic chalcogenide-based functionality, with the ultimate goal of an all-thin-film chip.  We already have all necessary ingredients.

5.	Design of an Ovonic thin-film-computer to operate flexibly in a conventional binary mode, an Ovonic multistate mode, or a hybrid mode for maximum commercial penetration.

6.	These remarkable advances in the conceptual structure of computers will necessitate rethinking of the theoretical foundations of computer science.  Focus should begin on the commercial implications of that transformation.

7.	Present emphasis in the semiconductor industry has been changing from increase in speed to architecture development.  With the ECD Ovonic chalcogenide architecture, speed increases can be achieved as well as increased functionality through architecture.  The increase in speed arises from both the great intrinsic speed of the Ovonic devices, their high intrinsic parallelism, and their capacity to scale down to the nanoscale and below without degradation of performance.  Their nonvolatility prevents the thermal budget from rising catastrophically as the scale is reduced.  Moreover, achievable current densities and currents remain so high that current proves no barrier to further miniaturization.

REFERENCES

1.  Ovshinsky, S.R., “Reversible electrical switching phenomena in disordered structures”, Phys.Rev.Lett. 21, 1450-1453, 1968.

2.  Cohen, M.H., Frtizsche, H., and Ovshinsky, S.R., “Simple band model for amorphous semiconducting alloys”, Phys.Rev.Lett. 22, 1065-1068, 1969.

3.  Kastner, M., Adler, D., Frtizsche, H., “Valence-alternation model for localized gap states in lone-pair semiconductors,” Phys.Rev.Lett. 37, 1504-1507, 1976.

4.  “Disordered materials, science and technology, Selected papers by S.R. Ovshinsky,” eds. Adler, D., Schwartz, B.B., and Silver, M., Plenum Press, New York, 1991.

5.  Ovshinsky, S.R., “The Ovonic cognitive computer - A new paradigm”, Proceedings of the Third European Symposium on Phase Change and Ovonic Science, E*PCOS 04, Sept. 6-7, Lichtenstein.

6.  The 17th International Flairs Conference had a special track on Neural Network applications for which papers on applications in the following areas were solicited:  Vision, Pattern Recognition, Control and Process Monitoring, Biomedical Applications, Speech Recognition, Text Mining, Diagnostic Problems, Telecommunications, Power Systems, Image Processing.  The URL is http://uhaweb.edu/irussell/st04.html.

7.  Martin, K., “Making your mind up”, Nature, 22 May 2003, 423, 383-384.

8.  Holler, M.; Tam, S.; Castro, H.; Benson, R.; “An electrically trainable artificial neural network (ETANN) with 10240 `floating gate' synapses”, Neural Networks, 1989. IJCNN, International Joint Conference on, 18-22 June 1989, Vol. 2, Pages: 191-196.

9.  Fritzsche, H., “Why chalcogenides are ideal materials for Ovshinsky’s Ovonic threshold and memory devices,” Physics and Chemistry of Glasses, 2004 (in press).

10.  Priovano, A., Lacaita, A.L., Benvenuti, A., Pellizer, F., Hudgens, S., and Bez, R., “Scaling analysis of phase-change memory technology”, IEEE 0-7803-7873, 2003.

11.  Cohen, M.H., “Scaling of amorphous semiconductor devices”, MRS Workshop on the Physics and Chemistry of Switching, San Francisco, April 1,2 (2005).

12.  Katz, B., “Nerve, muscle, and synapse,” London, McGraw-Hill, Inc., 1966. Page 3.

13.  Moerland, P.D. and Fiesler, E., “Neural network adaptations to hardware implementations”, in Handbook of Neural Computation, IOP Publishing Ltd and Oxford University Press, 1997.

14.  Lawrence, P.N., “Correlithm object technology”, Correlithm Publications, Dallas, 2004.